Arab American University

Department of Engineering

Project Proposal

Solar Guardian: An AI-Based Smart Solar Energy System with Fault Detection, Sun Tracking, and Consumption Monitoring

| Student Name | ID |
| --- | --- |
| Bisan Karem Mahmoud Qalalwe | 202210630 |
| Bana Mustafa Nasser Hamarshi | 202210761 |
| Dalya Yousef Saber Mustafa | 202210223 |

Supervisor Name: Dr.Hussein Younis

EXECUTIVE SUMMARY

Problem statement

Solar energy has become an important alternative source of electricity, particularly in regions such as Palestine where the public grid can be unreliable and costly. However, most small-scale solar installations only measure and display raw values such as voltage, current, and power. They do not tell the user whether the system is performing as expected, or why its output has dropped. Gradual losses caused by problems such as soiling, partial shading, or abnormal heating may go unnoticed until they result in significant energy loss or equipment damage, and diagnosing them requires time and technical expertise. In addition, fixed solar panels cannot follow the sun during the day, which limits the amount of solar radiation they capture. Finally, users of grid-connected or hybrid systems lack a continuous and clear view of their electricity consumption and of the amount they are expected to pay to the electricity company, and usually have to refer to the company to find out. Together, these gaps reduce energy efficiency, increase maintenance effort and dependence on the grid, and make it difficult for users to manage their energy and costs. There is therefore a need for an intelligent system that interprets the data of a solar installation, detects abnormal behavior early, increases energy capture, and presents the information to the user in a clear and actionable way.

Objectives of the study

The main objective of the Solar Guardian project is to develop a smart solar system that improves energy production, monitors system performance, and helps users understand their electricity consumption and costs.

The study aims to achieve the following specific objectives:

1. Fault Prediction and Detection: To monitor solar panel performance and identify potential faults or abnormal conditions, such as dust accumulation, partial shading, and unusual temperature changes, before they significantly affect energy production.

2. Automatic Sun Tracking: To develop a mechanism that automatically adjusts the solar panel's orientation to follow the sun throughout the day, allowing the panel to capture more sunlight and improve energy production compared to a fixed panel.

3. Electricity Consumption and Cost Calculation: To monitor electricity consumption and calculate its actual or estimated financial cost based on the applicable electricity tariff, providing users with a clear understanding of their energy usage and expenses.

Proposed solution

Solar Guardian is an integrated smart solar energy system designed to improve the monitoring and management of solar energy use. It combines AI-based solar panel performance analysis and potential fault detection with an automatic sun-tracking mechanism to support improved energy generation. The system also provides a mobile application through which users can monitor energy production, electricity consumption, and estimated electricity costs. By integrating these functions, Solar Guardian aims to provide users with a clearer understanding of their solar energy system and energy usage.

Main components: - Hardware: sensors, a microcontroller , local storage, and a sun-tracking mechanism. - AI: expected-power model, anomaly detection, and fault classification with severity, confidence, and recommendations. - Software: mobile application / dashboard for monitoring, alerts, history, consumption, and estimated bill.

1. Controller and Sensors

The controller acts as the main unit responsible for collecting and managing data from the sensors. The sensors will measure important parameters, including solar panel voltage, current, panel temperature, ambient temperature, and solar irradiance. The collected measurements will be used to monitor the panel's performance and control the sun-tracking mechanism, allowing the panel to adjust its orientation according to the sun's position.

2. Historical Data Storage

The system will store collected measurements and system readings over time in a local database. This historical data will allow users to review previous performance, compare energy production across different periods, and monitor daily, weekly, and monthly electricity consumption. It will also provide stored data for analyzing performance changes and identifying unusual patterns.

3. Artificial Intelligence (AI)

The AI component will analyze the collected sensor measurements and historical data to evaluate the solar panel's performance. It will estimate the expected power generation under different environmental conditions and compare it with the actual generated power to identify abnormal performance and potential faults, such as dust accumulation or partial shading. It will also help classify detected abnormalities and provide users with alerts and recommendations.

Expected outcomes

The project is expected to deliver an integrated solar energy monitoring system consisting of the following components:

1. Controller and Sensors System

A working hardware prototype consisting of a controller connected to sensors that measure voltage, current, panel temperature, ambient temperature, and solar irradiance. The controller will collect and process these measurements, monitor the panel's performance, and control the motorized mechanism responsible for adjusting the panel's angle to follow the sun.

2. Mobile Application

A user-friendly mobile application connected to the solar system, allowing users to monitor the system remotely. The application will display real-time measurements, energy production, panel status, detected faults and alerts, historical data, electricity consumption, and estimated costs.

3. Integrated System Operation

A complete working system in which the controller collects sensor data, stores historical readings, and sends relevant information to the mobile application. The system will analyze performance, identify abnormal conditions, provide alerts and recommendations, automatically adjust the panel's orientation, and help users track their electricity consumption and estimated financial costs.

Significance of the study

Solar Guardian aims to contribute to the renewable energy sector by providing an integrated approach to solar energy monitoring and management. Unlike solutions that focus on only one aspect of solar energy systems, the project brings together solar panel performance monitoring, AI-based analysis, automatic sun tracking, and electricity consumption monitoring. It aims to help users move beyond viewing basic electrical measurements toward understanding the performance and operating condition of their solar panels. The AI-based analysis is intended to identify abnormal performance and provide useful information about potential problems. The tracking mechanism is designed to adjust panel orientation throughout the day to support better solar energy collection. In addition, the mobile application will present electricity consumption information and estimated financial costs in an understandable format. This can help users make more informed decisions about their energy usage and expenses. The project is particularly relevant to the Palestinian context, where users may benefit from better monitoring of solar energy systems and greater awareness of their dependence on electricity supplied by the public grid. Its main contribution lies in integrating these complementary functions into one accessible system rather than introducing each technology as an entirely new concept.

Stackholders

| Category | Stakeholder | Role and interest |
| --- | --- | --- |
| Primary | Owners of solar / hybrid systems (households, small businesses) | Use the mobile app daily to follow system health, receive alerts and recommendations, and view consumption and the estimated electricity cost. Interest: higher energy yield, fewer unexpected failures, lower and more predictable costs. |
| Primary | Solar installers and maintenance technicians | Use the fault diagnosis, severity/confidence values and recommendations to locate problems faster and avoid unnecessary site visits. |
| Secondary | Household members and building occupants | Benefit indirectly from more reliable power and clearer awareness of electricity usage and cost. |
| Secondary | Local community and renewable energy sector | Benefit from longer equipment life, less wasted energy, and greater trust in adopting solar systems. |
| Other | Electricity distribution companies | Their tariffs are the reference for the bill estimate. Interest: consumers who understand and better manage their consumption. |
| Other | Energy regulators and standards bodies (e.g., the Palestinian Energy and Natural Resources Authority) | Set rules for solar installations and grid connection; the system must remain compatible with them. |

Target users

Households with rooftop solar or hybrid (solar + grid) systems in Palestine: homeowners who need to know whether the system is healthy and how much electricity they use and are expected to pay for, without technical expertise. Small businesses, shops, workshops and farms that rely on solar power: an unnoticed fault or lost production directly costs them money and operating time. Solar installation and maintenance companies: can use the system as a diagnostic aid when following up on customers' systems.

Environment

The system is installed at the solar installation: the sensors, controller and tracking mechanism sit with the panel outdoors, and the user interacts with it through a mobile application or dashboard. During the project it will be developed and tested on a small-scale setup (a single panel or a small string) in a real outdoor or outdoor-laboratory environment. The system is designed to be low cost and to make its core decisions locally, so that it remains useful where internet connectivity is not reliable.